| Experiment Proposal: AR_2013_No10 |  |
| --- | --- |
| Title | Author/Spokesperson |
| Test of MWPC Saturation Effects and Profile Distortions | A. Reiter (1431) B. Walasek-Höhne (1733) |
| Summary & Aim |  |
| - R&D for FAIR: Test of MWPC Saturation and Gas Gain Distortion - Investigation of Beam Profile Measurements - Measurement of Gas Gain dependency on chamber high voltage - The measurements will be performed with slow extraction from SIS18. - Requested beam time: 2 shifts - Required detectors HTPDI1I and HTPDG1G, Reference screen on target ladder - Recorded data: Ionisation chamber current and MWPC profiles as function of particle number in spill |  |
| Machine parameters |  |
| Machine; SIS18, slow extraction. Mode; B-exp. Exp. area; HTP. Ion species; Light, medium and heavy ions (nitrogen, carbon, iron, uranium). Beam energy; 300 – 800 MeV/u, in principle any. Spill length; Slow: 300 ms – 1000 ms. Particle number; 104 – 109 per spill. Repetition rate; ~ 0.1 Hz or higher. Shifts; 2 shift, other measurement in parallel mode. Beam Time Period; Autumn 2013, sprint 2014. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of HTP line, the following series of data are taken: - Signals of HTPDI1I and HTPDG1G as function of particle number in spill - Investigation of IC and MWPC linearity, stability of beam profiles - Reference beam profile derived from Cromox screen on target ladder |  |

Experiment Setup

Exp. area; HTP beam line, dump area. Description of setup; - BD components in chamber DK1 - Screen on target ladder behind last diagnostics chamber. Duration of setup; - 2013/2014 measurement campaign. DAQ & Electronics Software; - GSI data acquisition for DK1 components - BeamView for screen on target ladder. Trigger; Slow extr.: Cycle start - cycle end.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations; A. Reiter; To be done based on published papers.. Mech. Workshop; Not required. Beam Line Installation. Electronics & DAQ; DAQ in Atomic Physics Container for BeamView GSI DAQ for HTPDI1I and HTPDG1G. Control System Integration. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

Investigate: - Linearity of signal, beam profile distortions, gas gain curve by comparison to reference detectors HTPDI1I and HTPDI1S