| Experiment Proposal |  |
| --- | --- |
| Title | Author/Spokesperson |
| Screen investigations with Slow extraction | A. Reiter (1431) B. Walasek-Höhne (1733) |
| Summary & Aim |  |
| - R&D for FAIR: Test of screens with protons and light ions - Investigation of several types of screen materials - The measurements will be performed with slow extraction from HIT synchrotron. - Requested beam time: 1 shifts - Measured quantities: Absolute signal, linearity - Test of DAQ & analogue electronics chain with FESA-compatible hardware - Recorded data: Signals as function of particle number, beam width |  |
| Machine parameters |  |
| Machine; HIT synchrotron, slow extraction. Mode. Exp. area; QA room. Ion species; Protons & Light ions. Beam energy; Proton: up to 220 MeV Carbon: up to 430 MeV/u. Spill length; Several seconds. Particle number; 104 – 109 per spill. Repetition rate; few Hz. Shifts; 1 shifts. Beam Time Period; Q3/Q4 2013. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of QA beam line, the following series of data are taken: - Screen signals (beam profiles) for different ion species and energies - Investigation of linearity, light yield stability |  |

Experiment Setup

Exp. area; QA room, screen target ladder after BAMS. Description of setup. Duration of setup; - Installation for experiment duration. DAQ & Electronics Software; FESA DAQ. Trigger; Acquire full spill or parts of spill.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations. Mech. Workshop. Beam Line Installation. Electronics & DAQ. Control System Integration. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

Investigate: - Linearity, absolute calibration, etc.