| Experiment Proposal: AR_2013_No2 |  |
| --- | --- |
| Title | Author/Spokesperson |
| Test of various types of SEM grids | A. Reiter (1431) B. Walasek-Höhne (1733) |
| Summary & Aim |  |
| - R&D for FAIR: Test of prototype SEM grids - Test of DAQ & analogue electronics chain: QFW + FESA-compatible hardware - The measurements will be performed with fast and slow extraction from SIS18. - Requested beam time: 3 shifts - Measured quantities: Beam profile, signal strength and linearity - Comparison to other profile measurements (screen, MWPC) - Recorded data: Signals as function of particle number, clearing high-voltage |  |
| Machine parameters |  |
| Machine; SIS18, slow and fast extraction. Mode; B-exp. Exp. area; HTP. Ion species; Light (nitrogen) to heavy ions. Beam energy; 300 – 800 MeV/u, in principle any. Spill length; Slow: 300 ms up to 1 second ; Fast: h=4 or h=1. Particle number; 104 – 109 per spill. Repetition rate; ~0.1 Hz or higher. Shifts; 3 shifts. Beam Time Period; Autumn 2013. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of HTP line, the following series of data are taken: - SEM grid response as function of particle number, beam energy, clearing high-voltage - Reference detector at HTP: Ionisation chamber HTPDI1I and SEM HTPDI1S or transformers RT and FCT - Test of readout electronics: Charge-to-frequency converter (QFW) with stretcher |  |

Experiment Setup

Exp. area; HTP, new setup of beam line with FAIR standard vacuum chamber.. Description of setup. Duration of setup; - Permanent installation. DAQ & Electronics Software; Long cable to Atomic Physics container, New FESA DAQ. Trigger; Acquire full spill.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations. Mech. Workshop. Beam Line Installation. Electronics & DAQ. Control System Integration. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

- Dynamic range, lower detection limit - Resolution of SEM grids (Comparison with other detectors) - Cross talk effects (clearing electrodes)