| Experiment Proposal: AR_2013_No5 |  |
| --- | --- |
| Title | Author/Spokesperson |
| Optical Transistion Radiation for Beam Imaging | A. Reiter (1431) B. Walasek-Höhne (1733) |
| Summary & Aim |  |
| - R&D for FAIR: Test of method of beam imaging for heavy ions - Feasibility of OTR in proton beam lines towards pBar target - Investigation of several types of OTR target foils - The measurements will be performed with fast and slow extraction from SIS18. - Requested beam time: 10 shifts - Measured quantities: Absolute signal, beam profile - Comparison with SEM grid and MWPC/IC - Test of DAQ & analogue electronics chain with FESA-compatible hardware - Recorded data: Signals as function of particle number |  |
| Machine parameters |  |
| Machine; SIS18, fast extraction. Mode; B-exp. Exp. area; HTP. Ion species; Light and heavy ions. Beam energy; 300 – 800 MeV/u, in principle any. Spill length; Slow: 300 ms – 1000 ms Fast: h=4 or h=1. Particle number; 104 – 109 per spill. Repetition rate; ~ 0.1 Hz or higher. Shifts; 10 shifts, split into 2 campaigns. Beam Time Period; Autumn 2013, sprint 2014. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of HTP line, the following series of data are taken: - Screen signals (beam profiles) for different ion species and energies - Investigation of linearity, light yield stability, beta-dependency, q2 dependency |  |

Experiment Setup

Exp. area; HTP beam line. Description of setup; - New FAIR prototype vacuum chamber. Duration of setup; - Chamber: permanent installation - OTR setup: May be removed from chamber. DAQ & Electronics Software; FESA DAQ in Atomic Physics container, New FESA DAQ. Trigger; Slow extr.: Cycle start - cycle end Fast extr.: Kicker, Flattop reached.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations; Not required. Mech. Workshop. Beam Line Installation. Electronics & DAQ. Control System Integration. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

Investigate: - Linearity, absolute calibration, etc. - Use foil at HTP entrance as stripping foil (measurements with two charge-states to test q2 dependency of light yield)