| Experiment Proposal: AR_2013_No6 |  |
| --- | --- |
| Title | Author/Spokesperson |
| Test of Segmented Ionisation Chamber in Air for pBar Target Monitoring | A. Reiter (1431) M. Witthaus |
| Summary & Aim |  |
| - R&D for FAIR: Test of method of beam imaging for 30 GeV protons - Feasibility of segmented air ionisation chamber at pBar target - The measurements will be performed with fast and slow extraction from SIS18. - Requested beam time: 5 shifts - Measured quantities: Absolute signal, beam profile - Comparison with SEM grid and MWPC/IC - Test of DAQ & analogue electronics chain with FESA-compatible hardware - Recorded data: Signals as function of particle number, extraction length |  |
| Machine parameters |  |
| Machine; SIS18, fast extraction. Mode; B-exp. Exp. area; HTP. Ion species; Light and heavy ions. Beam energy; 300 – 800 MeV/u, in principle any. Spill length; Slow: 300 ms – 1000 ms Fast: h=4 or h=1. Particle number; 104 – 109 per spill. Repetition rate; ~ 0.1 Hz or higher. Shifts; 10 shifts, split into 2 campaigns. Beam Time Period; Autumn 2013, sprint 2014. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of HTP line, the following series of data are taken: - Chamber signals (beam profiles) for light/heavy ion species and different energies - Investigation of linearity, light yield stability, dE/dx dependency |  |

Experiment Setup

Exp. area; HTP beam line, dump area. Description of setup; - Pneumatic actuator at end of HTP beam line. Duration of setup; - 2013/2014 measurement campaign - No interference with other detectors, if in outer end position. DAQ & Electronics Software; FESA DAQ in Atomic Physics container, new FESA DAQ (QFW). Trigger; Slow extr.: Cycle start - cycle end Fast extr.: Kicker, Flattop reached.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations; Not required. Mech. Workshop; 3 days; Pneumatic actuator at HTP (special for T-DG090) Adaptation for mounting of detector in drive. Beam Line Installation; 0.5 days. Electronics & DAQ; DAQ in Atomic Physics Container, QFW with FESA DAQ system and GUI. Control System Integration; Not needed! Existing pneumatic actuator control.. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

Investigate: - Linearity, comparison with SEM grid and MWPC profiles