| Experiment Proposal: AR_2013_No7 |  |
| --- | --- |
| Title | Author/Spokesperson |
| Plastic Scintillator Response & Test of High-Rate Photomultipliers for FAIR beam loss system | A. Reiter (1431) T. Hoffmann |
| Summary & Aim |  |
| - R&D for FAIR: Test of HW for beam loss monitoring - Development of high-rate PMT - Scintillator response to minimum- and highly-ionising radiation (core & halo components) - The measurements will be performed with slow extraction from SIS18. - Requested beam time: 1 shifts - Measured quantities: Absolute signal, rate stability of PMT output, pulse-height spectra - Test of DAQ & analogue electronics chain with FESA-compatible hardware - Recorded data: Signals as function of particle type, count rate and PMT high voltage |  |
| Machine parameters |  |
| Machine; SIS18, slow extraction. Mode; B-exp. Exp. area; HTP. Ion species; Light, medium and heavy ions (nitrogen, carbon, iron, uranium) Light ions for rate test!. Beam energy; 300 – 800 MeV/u, in principle any. Spill length; Slow: 300 ms – 1000 ms. Particle number; 104 – 109 per spill. Repetition rate; ~ 0.1 Hz or higher. Shifts; 3 shift, other measurement in parallel mode. Beam Time Period; Autumn 2013, sprint 2014. Health & Safety; No concerns. |
| Experiment procedure |  |
| After setup of beam at end of HTP line, the following series of data are taken: - Scintillator signals for light to heavy ion species and different energies - Investigation of linearity, light yield stability, dE/dx dependency |  |

Experiment Setup

Exp. area; HTP beam line, dump area. Description of setup; - Vertical stepper-motor actuator at end of HTP beam line. Duration of setup; - 2013/2014 measurement campaign - No interference with other detectors, if in outer end position. DAQ & Electronics Software; FESA DAQ in Atomic Physics container, new FESA DAQ (QFW). Trigger; Slow extr.: Cycle start - cycle end.

Experiment Preparation / Required support Estimated amount of time, manpower and equipment

Estimates or simulations; Done. Mech. Workshop; 3 days; Pneumatic actuator at HTP Adaptation for mounting of detector in drive. Beam Line Installation; 0.5 days. Electronics & DAQ; DAQ in Atomic Physics Container, QFW with FESA DAQ system and GUI NEW FESA class required. Control System Integration; Not needed! New stepper motor hardware for FAIR including remote operation via GUI.. On-site tests. Modification of exp. area. Dismantling of setup.

Remarks & Comments

Investigate: - Linearity, dEdx dependency,